# REUSING EXISTING FOUNDATION

# IN EARTHQUAKE AFFECTED AREAS

Earthquake damaged superstructures of buildings in most cases. Except the cases where integrity of foundations may have been affected due to landslides, liquefaction or, any displacement of foundation stones the foundations are mostly undamaged and intact. These foundations can be reused for reconstruction as it is not only cost-effective but also more environment friendly. To re-use the foundation , following steps may be adopted.

| Steps | Description |
| --- | --- |
| 01 |  |
| Remove the debris around the foundation and clean the surface of the masonry and visually inspect the masonry close to the ground (approximately upto 1’-0” above the ground) for structural cracks. |  |
| 02 |  |
| Follow safety standards and remove rubble, salvage good stone, timber and other elements, stack and store. Remove upto ground level. |  |
| 03 | 3’ 3’ 3’ 3’ |
| In order to inspect the cross section of the foundation dig a pit (having a ground area of 3'×3' along the foundation on the outer edge). Check the type of soil it is resting on and ensure it is good hard soil and not soft clayey soil. Foundation depth should not be less than 2'-6" and width not less than 2’ for single storey house. Check with the traditional norms of setting the depth and width of foundation as they would be mostly based on soil type. Check if the foundation is sufficiently away from mountain slope or retaining wall. |  |
| 04 |  |
| The top layer of the masonry (varying between 3" to 6", depending on the type and size of stone in use) from the existing masonry at the ground level need to be removed. |  |
| 05 |  |
| Clean the surface of the masonry to remove all the dust, dirt and mud etc. from the top of the remaining masonry foundation and fill any voids if seen in the foundation. |  |
| 06 | 3’ (approx.) 3’ (approx.) 3’ (approx.) 3’ (approx.) 3’ (approx.) 3’ (approx.) |
| Remove a stone at an interval of approximately 3’ from the center of the foundation. Add vertical reinforcements (wooden or, steel), both at corners and at regular intervals, at appropriate level as per technical advice. |  |
| 07 | 3’ (approx.) 3’ (approx.) 3’ (approx.) 3’ (approx.) |
| Insert vertical stone into the existing masonry at an interval of approximately 3', which will work as shear key between old and new course of stone masonry. |  |
| 08 |  |
| Continue wall masonry above this level upto plinth level. Use large corner stones and through stones. |  |
| 09 | GI Containment Reinforcement System with 2mm dia GI wires for cross links, 4 mm dia GI wires for vertical containment reinforcement and GI Welded Wire Mesh (WWM) for horizontal Bands GI Containment Reinforcement System with 2mm dia GI wires for cross links, 4 mm dia GI wires for vertical containment reinforcement and GI Welded Wire Mesh (WWM) for horizontal Bands RCC Band System with vertical bars at corners and door/ window jambs RCC Band System with vertical bars at corners and door/ window jambs Wooden Band System with vertical wooden posts at corners Wooden Band System with vertical wooden posts at corners |
| Continue with construction starting with a plinth band at appropriate level and integrating with other elements of earthquake resistant system (wooden band system, RCC Band system or, GI CR system) as applicable |  |

United Nations Development Programme

Comprehensive Disaster Risk Management Program

Lalitpur, Nepal. Ph. No: 015529545